# F: Computational Fluid Dynamics — Mesh Generation

The simulation uses a quarter-symmetry rectangular domain (two symmetry planes at Y = 0 and Z = 0) spanning x ∈ [−18, 42] m with a 12 m transverse half-extent — approximately 6 body lengths upstream, 14 downstream, and 79 body radii transversely. The background block mesh consists of 10 × 51 × 10 cells.

Six nested refinement volumes are defined in snappyHexMesh to transition cell size from the far field to the wall (Table XX). The parameter nCellsBetweenLevels = 4 prevents abrupt size jumps. All wall surfaces are uniformly refined to level 10. Feature edges are extracted at includedAngle = 180° (3 144 edges, including the smooth ogive) and assigned level 9 to constrain snapping at the nosecone shoulder, fin edges, and boattail shoulder.

| Object | Geometry | Level | Purpose |
| --- | --- | --- | --- |
| Outer cylinder | r = 2 m, x ∈ [−9, 18] m | 2 | Far-field transition |
| Inner cylinder | r = 1 m, x ∈ [−3.6, 9] m | 4 | Bow shock, expansion fan, body shear layer |
| Stagnation sphere | r = 0.15 m, centre (0,0,0) | 8 | Bridges level-4 to level-10 (avoids cell banding) |
| Wake cylinder | r = 0.18 m, x ∈ [2.8, 5.5] m | 9 | Separated wake and fin trailing-edge shear layers |
| Wall surfaces | walls, fins, cone, tail | 10 | Foundation for boundary layer extrusion |
| Feature edges | Aconcagua.eMesh (3 144 edges) | 9 | Nosecone shoulder, fin edges, boattail shoulder |

Table XX: snappyHexMesh refinement hierarchy.

Seven prismatic layers are extruded over all walls with a first-layer thickness of 1 × 10⁻⁴ m and expansion ratio 1.3 (~1.7 mm total). Global quality limits are maxNonOrtho = 60°, maxSkewness = 4.0, and maxAspectRatio = 11. Relaxed thresholds (maxNonOrtho = 75°, maxSkewness = 6.0) apply during layer extrusion only. The domain is decomposed into 16 sub-domains using the Scotch algorithm.

[ FIGURE PLACEHOLDER: Whole mesh — quarter-domain overview ]

Fig. XX: Full quarter-domain mesh showing nested refinement zones.

[ FIGURE PLACEHOLDER: Nosetip mesh — stagnation-cap level-8 to level-10 transition ]

Fig. XX: Stagnation-cap detail.

[ FIGURE PLACEHOLDER: Nosecone mesh — ogive and shoulder, level-4 / 8 / 10 zones ]

Fig. XX: Nosecone mesh.

[ FIGURE PLACEHOLDER: Fins mesh — leading edge, trailing edge, and fin-root junction ]

Fig. XX: Fin surface mesh.

[ FIGURE PLACEHOLDER: Wake mesh — near-wake region downstream of boattail ]

Fig. XX: Near-wake mesh.